# Exercises KJM3110 Electrochemistry

Solutions are provided as Hidden text: File, Options, Display, Hidden text.

## Chapter 1 Electricity

A. Symbols and units. What are symbols and units for

Charge		q or Q 			C

Force 		f or F			N

Potential	φ			V

Voltage	U or E (not V)		V

Current 	I			A

Capacitance	C			F

Resistance 	R			ohm

Resistivity	ρ			ohm m

Conductance 	G			S

Conductivity	σ			S/m

B. Geometry. A tube with an electrolyte has inner diameter 2 cm and fully reversible electrodes covering the entire area, 10 cm apart. The electrolyte has a conductivity of 0.05 S/cm. What is its resistivity, conductance, and resistance? A voltage of 1.5 V is applied over the electrodes. What is the current and current density? If the charge carriers are singly charged (e.g. protons and chloride ions like in HCl(aq), what is the force they feel?)

Area A = (2 cm / 2)2 * 3.14 = 3.14 cm2. Length l = 10 cm.

ρ = 1/σ = 1 / 0.05 S/cm = 20 ohm cm.

G = σ A/l = 0.05 S/cm * 3.14 cm2 / 10 cm = 0.0157 S.

R = 1/G = 63.7 ohm.

I = U/R = 1.5 V / 63.7 ohm = 0.0245 A.

i = I/A = 0.0235 A / 3.14 cm2 = 0.00748 A/cm2.

Field X = U/l = 1.5 V / 10 cm = 1.5 V / 0.1 m = 15 V/m. F = X q = 15 V/m * 1.602⋅10-19 C = 2.4⋅10-18 N.

C. Electrostatic forces.

i) Calculate the classical electrostatic force between a proton and an electron 1 Å away.

F = -1.609∙10-19 C * -1.609∙10-19 C / (4*3.14*8.854∙10-12 F/m*(1∙10-10 m)2) = 2.33∙10-8 N

ii) Calculate the electrostatic force between a K+ ion and an O2- ion 2.8 Å away.

F = -1.609∙10-19 C * -2* 1.609∙10-19 C / (4*3.14*8.854∙10-12 F/m*(2.8∙10-10 m)2) = 5.94∙10-9 N

iii) Calculate the electrostatic force between a H3O+ ion and an OH- ion 30 Å away in vacuum and in water.

Vacuum:

F = -1.609∙10-19 C * -1.609∙10-19 C / (4*3.14*8.854∙10-12 F/m*(30∙10-10 m)2) = 2.6∙10-11 N

Water (relative permittivity = 80):

F = -1.609∙10-19 C * -1.609∙10-19 C / (4*3.14* 80* 8.854∙10-12 F/m*(30∙10-10 m)2) = 3.2∙10-13 N

D. Field.

i) What is the field strength 1 nm away from an O2- ion (in vacuum)?

X = -(-2* 1.609∙10-19 C) / (4*3.14*8.854∙10-12 F/m * (1∙10-9 m)2) = 2.88∙109 V/m

ii) We have two large plates 1 mm apart in vacuum. They have areal charge densities of 1 C/m2. What is the field strength between the plates?

X = 1 C/m2 / 8.854∙10-12 F/m = 1.13∙1011 V/m

E. Capacitor.

i) A capacitor has area 1 cm2 and a dielectric material of thickness 10 micrometer and a relative dielectric constant of 100. What is its capacitance?

C = 100 * 8.854∙10-12 F/m * 1 cm2 / (100 cm/m)2 / (10 μm / 106 μm/m) = 8.854∙10-9 F = 8.854 nF.

ii) A voltage of 10 V is applied over it. How much charge will it take (can it store)?

q = CU = 8.854∙10-9 F * 10 V = 8.854∙10-8 C

iii) We discharge the capacitor over a load without other losses. What is the energy dissipated? (This is beyond curriculum, but gives opportunity for reflection. Look up the derivation or formula.) Is the energy the same as it took to charge it? If there were losses, what would be their causes?

The total energy is obtained by integrating over time, current, or charge, during charge or discharge and comes out as ½ CU2 = ½ * 8.854∙10-9 F * (10 V)2 = 4.427∙10-7 J. It is in principle the same during charge and discharge. Losses come from (parasitic) series resistance in wires and contacts, and in slowness in reorientation of dipoles in the dielectric medium. (In addition, we can have loss of charge due to imperfect insulation (parasitic parallel conductance).

F. Conductivity

i) A material has a conductivity of 0.01 S/cm. What is its resistivity?

ρ = 1/σ = 100 ohm cm

ii) A thin sample of the material 0.01 mm thick has an area of 10 cm2 and is equipped with electrodes covering the whole area. What is the conductance and resistance of the component?

G = σ A / l = 0.01 S/cm * 10 cm2 / (0.01 mm / 10 mm/cm) = 100 S

R = 1/G = 0.01 ohm

iii) We apply a voltage of 1 V over the electrodes. What is the current? What is the current density?

I = 1 V / 0.01 ohm = 100 A

i = 100 A / 10 cm2 = 10 A/cm2

G. Mobility

A solid electrolyte material has a concentration of mobile oxide ions (O2-) of 1∙1020 /cm3 and they have a charge mobility of 1∙10-4 cm2/Vs. What is the ionic conductivity of the material?

σ = ze c u = 2 * 1.602∙10-19 C * 1∙1020 /cm3 * 1∙10-4 cm2/Vs = 0.0032 S/cm

H. Electronic and ionic conductivity

i) A material has an electronic conductivity of 0.001 S/cm and an ionic conductivity of 0.01 S/cm. What is the total conductivity? What is the ionic transport number? What is the electronic transport number? What is the total transport number?

Total conductivity = 0.0001 S/cm + 0.01 S/cm = 0.011 S/cm. Ionic transport number = 0.01/0.011 = 0.909 = 90.9%. Electronic transport number = 0.001/.011 = 0.0909. Total transport number ≡ 1.

ii) If you consider that the material is also a dielectric (has a relative dielectric constant >1) discuss a possible circuit schematic for the material, and for the material connected to a voltage source, voltmeter, and amperemeter.

Parallel circuit of two resistors and a capacitor inserted for the cell in figures in Ch. 1 of the textbook.